REPORT

注意事項: 當天demo只需填寫Experiment Result與 Waveform，剩下部分由同學回去自行填寫

Experiment 1: Difference Amplifier

請參考講義 “109電工學實驗 Lab07 ver 1.pdf” 第8頁

Write down your pinout. (示波器的通道)

2. Differential Mode

|  | v1,pp (V) | v2,pp (V) | vd,pp=v2-v1 (V) | vout,pp (V) | ADM (V/V) | Phase (vout->v1) (degree) |
| --- | --- | --- | --- | --- | --- | --- |
| Theoretical Result | 2 | 0 | -2 | 2 | -1 | 180 |
| Experiment Result | 2.08 | 0.08 | -2 | 2.08 | -1.04 | 177.9 |
| Simulation Result |  |  |  |  |  |  |
| Error (%) | 4 | ∞ | 0 | 4 | 4 | -1.17 |

P1_vout , v2 and v1 experiment waveform (一張圖同時呈現三通道的波型，用Meas 顯示Phase)

Simulation:

P2_LTspice電路圖

P3_vout , v2 and v1 simulation waveform

3. Common Mode

|  | v1,pp (V) | vout,pp (V) | ACM (V/V) | Phase (vout->v1) (degree) |
| --- | --- | --- | --- | --- |
| Theoretical Result | 10 | 0 | 0 | 0 |
| Experiment Result | 10.2 | 0.0176 | 0.001725 | 11.1 |
| Simulation Result |  |  |  |  |
| Error (%) | 2 | ∞ | ∞ | ∞ |

P4_vout , v2 and v1 experiment waveform (一張圖同時呈現三通道的波型，用Meas 顯示Phase)

Simulation:

P5_LTspice電路圖

P6_vout , v2 and v1 simulation waveform

4. CMRR

Theoretical CMRR=ADMACM=-10=∞

Experiment CMRR=ADMACM=-1.040.001725=602.90

Simulation CMRR=ADMACM=

5.Discuss:

In experiment 1, R1=R2=R3=R4. Therefore, the gain should be -R2R1=-1. In the first graph, we can see that the gain is -1.04, with an error of 4%. Also, because the waveform is inverted, the phase of vout→v1 should be 180°, and we have an experiment result of 177.9° with an error of 1.17%.

In experiment 2, v1 and v2 is connected. Therefore, the waveform is identical. Since v2-v1=0, the output voltage should be approximately 0. We obtain a common mode amplitude of 0.001725, which is close to 0.

The CMRR is ∞ in theory since ACM is 0. We obtain an experiment CMRR of 602.90.

Experiment 2: Instrumentation Amplifier

請參考講義 “109電工學實驗 Lab07 ver 1.pdf” 第14頁

Write down your pinout. (示波器的通道)

2. Differential Mode

|  | v1,pp (V) | v2,pp (V) | vd,pp=v2-v1 (V) | vout,pp (V) | ADM (V/V) | Phase (vout->v1) (degree) |
| --- | --- | --- | --- | --- | --- | --- |
| Theoretical Result | 0.02 | 0 | -0.02 | -2.02 | 101 | 180 |
| Experiment Result | 0.0296 | 0.00840 | -0.0212 | -2.16 | 101.89 | 175.1 |
| Simulation Result |  |  |  |  |  |  |
| Error (%) | 48 | ∞ | 6 | 6.93 | 0.88 | -2.72 |

P7_vout , v2 and v1 experiment waveform (一張圖同時呈現三通道的波型，用Meas 顯示Phase)

Simulation:

P8_LTspice電路圖

P9_vout , v2 and v1 simulation waveform

3. Common Mode

|  | v1,pp (V) | vout,pp (V) | ACM (V/V) | Phase (vout->v1) (degree) |
| --- | --- | --- | --- | --- |
| Theoretical Result | 10 | 0 | 0 | 0 |
| Experiment Result | 10.2 | 1.4 | 0.14 | 14.5 |
| Simulation Result |  |  |  |  |
| Error (%) | 2 | ∞ | ∞ | ∞ |

P10_vout , v2 and v1 experiment waveform (一張圖同時呈現三通道的波型，用Meas 顯示Phase)

Simulation:

P11_LTspice電路圖

P12_vout , v2 and v1 simulation waveform

4. CMRR

Theoretical CMRR=ADMACM=1010=∞

Experiment CMRR=ADMACM=101.890.14=727.79

Simulation CMRR=ADMACM=

5.Discuss:

The theoretical result is similar to experiment 1, except the differential mode amplitude is 101.Also, the experiment result is approximately equal to the theoretical result, except v1 of first figure. This is because the input wave has vpp=20 mV, and our wave generator cannot generate a precise waveform of this small.

Although we cannot generate the small precise waveform, we still obtain ADM with an error of 0.88%.

Experiment 3: Difference Amplifier

Write down your pinout. (示波器的通道)

2. Differential Mode (R1 = 100Ω, R2 = 1kΩ)

|  | v1,pp (V) | v2,pp (V) | vd,pp=v2-v1 (V) | vout,pp (V) | ADM (V/V) | Phase (vout->v1) (degree) |
| --- | --- | --- | --- | --- | --- | --- |
| Experiment Result | 1.38 | 0.0096 | -1.3704 | 13 | -9.49 | -179.3 |

3. Differential Mode (R1 = 10kΩ, R2 = 100kΩ)

|  | v1,pp (V) | v2,pp (V) | vd,pp=v2-v1 (V) | vout,pp (V) | ADM (V/V) | Phase (vout->v1) (degree) |
| --- | --- | --- | --- | --- | --- | --- |
| Experiment Result | 2.08 | 0.0056 | -2.0744 | 20 | -9.64 | -179.1 |

4. Compare the difference between the two difference amplifiers, and explain why.

By observing, we can find out that the amplitudes of two experiment are similar, but the resistors we used in two experiments are different.

This is because the ratio R_2/R_1 for both experiment is 10. Although we have different resistors, as long as the ratio of resistors are the same, we will obtain identical results.