REPORT

Experiment 1: LED Blink.

NOTE: use DC coupling to observe the waveforms, and use measure to mark Vpp and frequency.

1.

Arduino D13 output waveform with frequency 0.5Hz.

2.

Arduino D4 output waveform with frequency 1Hz.

Question:

What output level (high/low) will make LED turn light or dark? Why?

High level will make LED turn light, and low level will make LED turn dark. High level means the LED

Experiment 2: LED Blink with push button.

1.

Is this circuit a pull-high or pull-low type? _pull-low type_.

2.

Measure pin D2 voltage using voltage meter:

| Behavior | DC Voltage (V) |
| --- | --- |
| Pushbutton pressed | 5.12 |
| Pushbutton released | 0.08 |

3.

Change your circuit to another type and modify your code to achieve the same requirement.

Modified circuit (photo):

4.

Measure pin D2 voltage using voltage meter (the modified circuit):

| Behavior | DC Voltage (V) |
| --- | --- |
| Pushbutton pressed | 0.16 |
| Pushbutton released | 5.12 |

5.

Attach your modified code (screen shot):

/*

*This lab is to combine example 2 and example 3 in lecture to achieve following requirement.

*When push-button is pressed, turn the LED on, and output “buttonState” to serial monitor.

*When push-button is released, make LED blink, and output “buttonState” to serial monitor.

*

*The circuit:

*LED A of 7-Segment attached from 1k resistor to pin 13

*1k resistor attached to LED A of 7-Segment from ground

*pushbutton attached to 1K resistor from +5V

*Note: on most Arduinos there is already an LED on the board attached to pin 13.

*

*/

int pushButton = 2;

int led = 13;

void setup() {

// initialize serial communication at 9600 bits per second:

Serial.begin(9600);

// make the pushbutton's pin an input and led’s pin an output:

pinMode(pushButton, INPUT);

pinMode(led, OUTPUT);

}

void loop() {

// read the input pin:

int buttonState = digitalRead(pushButton);

// make LED blink and outputbuttonState.

if(buttonState == HIGH){

digitalWrite(led, LOW);// turn the LED on(HIGH is　the voltage level)

delay(1000); // wait for　a　second

digitalWrite(led,HIGH);//turn　the LED off by　making the　voltage　LOW

delay(1000);

Serial.println(buttonState);

}

else{//turn LED on and　output　buttonState.

digitalWrite(led, LOW);

Serial.println(buttonState);

delay(2000);

}

}

Experiment 3: student ID Marquee.

Attach your code and circuit (photo):

:

/*

*This lab is to use one 1k Ω resistor, one push-button,

*eight 1k Ω resistors, one 7-Segment display,

*and one Aruduino platform to implement LED marquee.

*The function is as follows.

*When push-button is pressed, turn all LEDs of 7-segment on.

*When push-button is released, it show LED marquee.

*

*The circuit:

*The pin assignments between 7-Segment and Arduino is as follows.

*(Remember to connect a 220 current-limit resistor between LED of 7-Segment and Arduino pin.)

*7-segment A B C D E F G DP

*Arduino 13 12 11 10 9 8 7 6

*pushbutton attached to 1K resistor from +5V

*

*/

byte pinA=13, pinB=12, pinC=11, pinD=10, pinE=9, pinF=8, pinG=7, pinP=6;

byte pushButton = 2;

void setup() {

pinMode(pinA, OUTPUT); pinMode(pinB, OUTPUT); pinMode(pinC, OUTPUT);

pinMode(pinD, OUTPUT); pinMode(pinE, OUTPUT); pinMode(pinF, OUTPUT);

pinMode(pinG, OUTPUT); pinMode(pinP, OUTPUT);

pinMode(pushButton, INPUT);

}

// show thespecifiedLEDsof7-segment

void showSevenSeg(byte A, byte B, byte C, byte D, byte E, byte F, byte G, byte P){

digitalWrite(pinA, A); digitalWrite(pinB, B); digitalWrite(pinC, C); digitalWrite(pinD, D); digitalWrite(pinE, E); digitalWrite(pinF, F); digitalWrite(pinG, G); digitalWrite(pinP, P);

}

void showSevenSeg(char chr)

{

if(chr == '0')         showSevenSeg(1, 1, 1, 1, 1, 1, 0, 0);

else if(chr == '1')    showSevenSeg(0, 1, 1, 0, 0, 0, 0, 0);

else if(chr == '2')    showSevenSeg(1, 1, 0, 1, 1, 0, 1, 0);

else if(chr == '3')    showSevenSeg(1, 1, 1, 1, 0, 0, 1, 0);

else if(chr == '4')    showSevenSeg(0, 1, 1, 0, 0, 1, 1, 0);

else if(chr == '5')    showSevenSeg(1, 0, 1, 1, 0, 1, 1, 0);

else if(chr == '6')    showSevenSeg(1, 0, 1, 1, 1, 1, 1, 0);

else if(chr == '7')    showSevenSeg(1, 1, 1, 0, 0, 0, 0, 0);

else if(chr == '8')    showSevenSeg(1, 1, 1, 1, 1, 1, 1, 0);

else if(chr == '9')    showSevenSeg(1, 1, 1, 1, 0, 1, 1, 0);

}

void showSevenSeg(int num)

{

showSevenSeg(char('0' + num));

}

void loop() {

// read the input pin:

int buttonState = digitalRead(pushButton);

if(buttonState == LOW) {

// write 109611066 to the 7-segment

showSevenSeg(1);    delay(500);

showSevenSeg(0);    delay(500);

showSevenSeg(9);    delay(500);

showSevenSeg(6);    delay(500);

showSevenSeg(1);    delay(500);

showSevenSeg(1);    delay(500);

showSevenSeg(0);    delay(500);

showSevenSeg(6);    delay(500);

showSevenSeg(6);    delay(500);

}

else{

showSevenSeg(1,1,1,1,1,1,1,1);// turn all LEDs of 7-segment on.

delay(100);

}

}